## Smart Water Quality Monitoring System

This section provides the full technical breakdown for my Smart Water Quality Monitoring System. Since you are using a Raspberry Pi 3, these details are critical because of the specific way the Pi handles analog vs. digital data.

## 1. System Components & Specifications

Each component has a specific role in satisfying your assignment's features (Contamination Detection, Usage Tracking, etc.).

| Component Name | Role / Function | Key Specification |
| --- | --- | --- |
| Raspberry Pi 3 | Central Brain & Dashboard | 1.2GHz CPU, 1GB RAM, Built-in Wi-Fi. |
| MCP3008 ADC | Analog-to-Digital Bridge | 10-bit resolution (0–1023), 8 channels, SPI interface. |
| pH Sensor | Acidity/Alkalinity Level | Range: 0.0–14.0 pH; Output: Analog Voltage. |
| TDS Sensor | Total Dissolved Solids | Range: 0–1000ppm; measures water "hardness." |
| Turbidity Sensor | Water Clarity | Measures suspended particles; Output: 0–4.5V. |
| Water Flow | Usage Tracking | Hall Effect sensor; 1–30L/min; Pulse-based. |
| DS18B20 | Temperature Monitoring | Digital (1-Wire); Range: -55°C to +125°C. |

## 2. Master Wiring Diagram (Pin-to-Pin)

This is how you must connect the wires in Fritzing to ensure the project works.

### The SPI Interface (MCP3008 to Pi)

The MCP3008 is required because the Pi cannot "understand" the analog signals from pH, TDS, and Turbidity directly.

- MCP3008 Pin 16 (VDD) & 15 (VREF)                              Pi Pin 1 (3.3V)

- MCP3008 Pin 14 (AGND) & 9 (DGND)                           Pi Pin 6 (GND)

- MCP3008 Pin 13 (CLK)                                                      Pi Pin 23 (GPIO 11)

- MCP3008 Pin 12 (DOUT)                                                  Pi Pin 21 (GPIO 9)

- MCP3008 Pin 11 (DIN)                                                      Pi Pin 19 (GPIO 10)

- MCP3008 Pin 10 (CS)                                                        Pi Pin 24 (GPIO 8)

### The Sensor Connections

#### Analog Inputs (To MCP3008)

Connect the "Signal" wire of these three sensors to the MCP3008:

- pH Signal                           CH0 (MCP3008 Pin 1)

- TDS Signal                         CH1 (MCP3008 Pin 2)

- Turbidity Signal                CH2 (MCP3008 Pin 3)

#### Digital Inputs (Direct to Pi)

- Water Flow (Pulse): Signal (Yellow)                   GPIO 27 (Pi Pin 13). Power with 5V.

- Temperature (DS18B20): Data (Yellow)             GPIO 4 (Pi Pin 7).

  - Note: You MUST add a 4.7kohm resistor between the Data wire and the 3.3V wire.

## 3. Alternative Options & Logic

- ADC Alternatives: If you cannot find an MCP3008, the ADS1115 is a higher-precision alternative (16-bit) that uses the I2C protocol instead of SPI.

- Logic Level Shifting: Most water sensors output 5V signals. While the MCP3008 can handle this if powered by 5V, ensure the output pin (DOUT) going to the Pi does not exceed 3.3V to protect your Pi's GPIOs.

- Calibration: * pH: Needs to be calibrated using buffer solutions (pH 4.0 and 7.0).

  - TDS: Needs a conversion factor (usually 0.5) to turn the raw voltage into "PPM" (Parts Per Million).

## 4. Final Verification

Before you "Submit Work" on your portal (as seen in your screenshot):

- Check for Common Ground: Is every GND wire connected to the Pi's GND? (If not, the readings will jump randomly).

- Enable Interfaces: Did you turn on SPI and 1-Wire in raspi-config?

- Resistor: Did you include the 4.7kohm resistor for the DS18B20?